DATA SHEET

MULTILAYER CERAMIC CAPACITORS

CC Series

X7R

16 V TO 100 V

Product specification – Aug 17, 2005 V.9 Supersedes Date of Jan 21, 2005

|  |  |  |  |  |  | 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

SCOPE

This specification describes Yageo CC X7R series chip capacitors.

ORDERING INFORMATION

Part number is identified by the series, size, tolerance, packing style, temperature coefficient, rated voltage and capacitance value.

CC × × × ×   ×   ×   X7R ×   BB × × ×

(1)	(2) (3)	(4)	(5)

(1) SIZE – INCH BASED (METRIC)

0402 (1005)

0603 (1608)

0805 (2012)

1206 (3216)

1210 (3225)

1812 (4532)

(2) TOLERANCE

J = ±5%

K = ±10%

(3) PACKING STYLE

R = 7” paper tape

K = 7” blister tape

P = 13” paper tape

F = 13” blister tape

C = Bulk case

(4) RATED VOLTAGE

7 = 16 V

8 = 25 V

9 = 50 V

0 = 100 V

(5) CAPACITANCE VALUE:

First two for significant figures and 3rd for number of zero

Letter “R” for decimal point

CONSTRUCTION

|  | terminations |  |
| --- | --- | --- |
|  | electrodes |  |
| Fig. 1 Surface mounted multilayer | MLB457 |  |
| ceramic capacitor construction | ceramic material |  |

DIMENSION

For dimension see Table 1

T	W

| Fig. 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface mounted multilayer ceramic |  |  |  |  |  |  |  | L 2 |  |  | L 4 |  |  |  | L 3 |  |  | MBB211 |  |  |
|  | capacitor dimension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | L 1 |  |  |  |  |  |  |  |  |  |
| Table 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TYPE |  | CC0402 | CC0603 | CC0805 |  |  |  |  |  | CC1206 |  |  |  | CC1210 |  |  |  |  |  | CC1812 |  |
| L1 (mm) |  | 1. 0±0.05 | 1.6±0.10 | 2.0±0.10 |  |  |  |  |  | 3.2±0.15 |  |  |  | 3.2±0.20 |  |  |  |  |  | 4.5±0.20 |  |
| W (mm) |  | 0.5±0.05 | 0.8±0.07 | 1.25±0.10 |  |  |  |  |  | 1.6±0.15 |  |  |  | 2.5±0.20 |  |  |  |  |  | 3.2±0.20 |  |
| T (mm) | min. | 0.45 | 0.73 | 0.50 |  |  |  |  |  | 0.50 |  |  |  | 0.50 |  |  |  |  |  | 0.50 |  |
|  | max. | 0.55 | 0.87 | 1.35 |  |  |  |  |  | 1.35 |  |  |  | 1.80 |  |  |  |  |  | 1.80 |  |
|  | min. | 0.15 | 0.20 | 0.25 |  |  |  |  |  | 0.25 |  |  |  | 0.25 |  |  |  |  |  | 0.25 |  |
| L2/L3 (mm) max. |  | 0.30 | 0.60 | 0.75 |  |  |  |  |  | 0.75 |  |  |  | 0.75 |  |  |  |  |  | 0.75 |  |
| L4 (mm) | min. | 0.40 | 0.40 | 0.55 |  |  |  |  |  | 1.40 |  |  |  | 1.40 |  |  |  |  |  | 2.20 |  |

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

CAPACITANCE RANGE & THICKNESS FOR 16V & 25V

| Table 2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAPACITANCE |  | 16 V |  |  |  | 25 V |  |  |  |  |
| (nF) |  | 0402 | 0603 | 0805 | 1206 | 0402 | 0603 | 0805 | 1206 | 1210 |
| 3.3 |  |  |  |  |  | 0.5 ±0.05 |  |  |  |  |
| 3.9 |  |  |  |  |  |  |  |  |  |  |
| 4.7 |  |  |  |  |  |  |  |  |  |  |
| 5.6 |  |  |  |  |  |  |  |  |  |  |
| 6.8 |  | 0.5 ±0.05 |  |  |  |  |  |  |  |  |
| 8.2 |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  | 0.8 ±0.07 | 0.6 ±0.1 |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  | 0.8 ±0.07 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  | 0.6 ±0.1 |  |  |  | 0.85 ±0.1 |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  |  |  |  |  |  |  |
| 82 |  |  |  | 0.85 ±0.1 |  |  |  |  |  |  |
| 100 |  |  |  |  |  |  |  |  | 0.85 ±0.1 |  |
| 120 |  |  |  |  |  |  |  |  |  |  |
| 150 |  |  |  |  |  |  |  |  |  |  |
| 180 |  |  |  |  |  |  |  |  |  |  |
| 220 |  |  |  |  | 0.85 ±0.1 |  |  |  |  | 0.85 ±0.1 |
| 270 |  |  |  |  |  |  |  |  | 1.15 ±0.1 |  |
| 330 |  |  |  | 1.25 ±0.1 |  |  |  |  |  |  |
| 390 |  |  |  |  | 1.15 ±0.1 |  |  |  |  |  |
| 470 |  |  |  |  |  |  |  |  |  |  |

560

680

820

1,000

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

CAPACITANCE RANGE & THICKNESS FOR 50V & 100V

| Table 3 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAPACITANCE |  | 50 V |  |  |  |  |  | 100 V |  |  |  |
| (nF) |  | 0402 | 0603 | 0805 | 1206 | 1210 | 1812 | 0805 | 1206 | 1210 | 1812 |
| 0.10 |  | 0.5 ±0.05 | 0.8 ±0.07 |  |  |  |  |  |  |  |  |
| 0.12 |  |  |  |  |  |  |  |  |  |  |  |
| 0.15 |  |  |  |  |  |  |  |  |  |  |  |
| 0.18 |  |  |  |  |  |  |  |  |  |  |  |
| 0.22 |  |  |  | 0.6 ±0.1 | 0.85 ±0.1 |  |  | 0.6 ±0.1 | 0.85 ±0.1 |  |  |
| 0.27 |  |  |  |  |  |  |  |  |  |  |  |
| 0.33 |  |  |  |  |  |  |  |  |  |  |  |
| 0.39 |  |  |  |  |  |  |  |  |  |  |  |
| 0.47 |  |  |  |  |  |  |  |  |  |  |  |
| 0.56 |  |  |  |  |  |  |  |  |  |  |  |
| 0.68 |  |  |  |  |  |  |  |  |  |  |  |
| 0.82 |  |  |  |  |  |  |  |  |  |  |  |
| 1.0 |  |  |  |  |  |  |  |  |  |  |  |
| 1.2 |  |  |  |  |  |  |  |  |  |  |  |
| 1.5 |  |  |  |  |  |  |  |  |  |  |  |
| 1.8 |  |  |  |  |  |  |  |  |  |  |  |
| 2.2 |  |  |  |  |  |  |  |  |  |  |  |
| 2.7 |  |  |  |  |  |  |  |  |  |  |  |
| 3.3 |  |  |  |  |  |  |  |  |  |  |  |
| 3.9 |  |  |  |  |  |  |  |  |  |  |  |
| 4.7 |  |  |  |  |  |  |  |  |  |  |  |
| 5.6 |  |  |  |  |  |  |  |  |  |  |  |
| 6.8 |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  | 0.85 ±0.1 |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  | 0.85 ±0.1 |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  | 0.85 ±0.1 |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  |  |  |  | 0.85 ±0.1 |  |

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

CAPACITANCE RANGE & THICKNESS FOR 50V & 100V (CONTINUED)

| Table 4 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAPACITANCE | 50 V |  |  |  |  |  | 100 V |  |  |  |
| (nF) | 0402 | 0603 | 0805 | 1206 | 1210 | 1812 | 0805 | 1206 | 1210 | 1812 |
| 56 |  |  | 0.85 ±0.1 | 0.85 ±0.1 | 0.85 ±0.1 |  |  | 1.15 ±0.1 | 0.85 ±0.1 |  |
| 68 |  |  |  |  |  |  |  |  |  |  |
| 82 |  |  |  |  |  |  |  |  |  |  |
| 100 |  | 0.8 ±0.07 |  |  |  | 1.15 ±0.1 |  |  |  | 1.15 ±0.1 |
| 120 |  |  |  |  | 1.15 ±0.1 |  |  |  | 1.15 ±0.1 |  |
| 150 |  |  |  | 1.15 ±0.1 |  |  |  |  |  |  |
| 180 |  |  |  |  |  |  |  |  |  |  |
| 220 |  |  |  |  |  |  |  |  |  |  |
| 270 |  |  |  |  |  |  |  |  |  |  |
| 330 |  |  |  |  |  |  |  |  |  |  |
| 390 |  |  |  |  |  |  |  |  |  | 1.6 ±0.2 |
| 470 |  |  |  |  |  |  |  |  |  |  |
| 560 |  |  |  |  |  | 1.6 ±0.2 |  |  |  |  |
| 680 |  |  |  |  |  |  |  |  |  |  |
| 820 |  |  |  |  |  |  |  |  |  |  |
| 1,000 |  |  |  |  |  |  |  |  |  |  |

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

THICKNESS CLASSES AND PACKING QUANTITY

| Table 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| THICKNESS | 8mm TAPE WIDTH / AMOUNT PER REEL |  |  |  |  | 12mm TAPE WIDTH / |  |  |  |  |  |  |
| CLASSIFICATION |  |  |  |  |  | AMOUNT PER REEL | AMOUNT PER BULK CASE |  |  |  |  |  |
| (mm) | Ø180mm, 7” |  | Ø330mm, 13” |  |  | Ø180mm, 7” Blister |  |  |  |  |  |  |
|  | Paper | Blister | Paper | Blister |  | 1812 | 0402 |  | 0603 | 0805 |  |  |
| 0.5 ±0.05 | 10,000 | --- | 50,000 | --- | --- |  | 50,000 | --- |  | --- |  |  |
| 0.6 ±0.10 | 4,000 | --- | 20,000 | --- | --- |  | --- | --- |  | 10,000 |  |  |
| 0.8 ±0.07 | 4,000 | --- | 15,000 | --- | --- |  | --- | 15,000 |  | --- |  |  |
| 0.85 ±0.10 | 4,000 | --- | 15,000 | --- | --- |  | --- | --- |  | 8,000 |  |  |
| 1.15 ±0.10 | --- | 3,000 | --- | 10,000 | --- |  | --- | --- |  | --- |  |  |
| 1.25 ±0.10 | --- | 3,000 | --- | 10,000 | --- |  | --- | --- |  | 5,000 |  |  |
| 1.6 ±0.20 | --- | --- | --- | --- | 1,000 |  | --- | --- |  | --- |  |  |

ELECTRICAL CHARACTERISTICS

| Table 5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CHARACTERISTICS | TEST CONDITIONS | REQUIREMENT |  |  |
| Operation temperature range | --- | –55 °C to +125 °C |  |  |
| Temperature characteristic/coefficient (TC) | With respect to 20 °C within operation | ±15% |  |  |
|  | temperature range |  |  |  |
| Capacitance tolerance | 1 Vrms/1 KHz at 20 °C | ±5%, ±10% |  |  |
| Dissipation factor (Tan d) | 1 Vrms/1 KHz at 20 °C | 25 V, 50 V & 100 V; ≤2.5% |  |  |
|  |  | 16 V; ≤3.5% |  |  |
| Insulation resistance (IR) | At Ur (rated voltage) for 1 minute | C ≤ 10 nF; Rins | ≥ 10 GΩ |  |
|  |  | C > 10 nF; Rins | x C ≥ 500 s |  |
| Dielectric withstanding Voltage | At 2.5 × Ur (for Ur ≤ 100V) | No breakdown |  |  |
|  | 1.5 × Ur + 100 V for 5 sec |  |  |  |

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

TAPING REEL

| Table 6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TAPE WIDE | 8mm | 8mm | 12mm |  | T |  |
| ØA (mm) | 180 | 330 | 180 |  |  |  |
| ØB (mm) | 62±1.5 | 62±1.5 | 62±1.5 |  |  |  |
| ØD (mm) | 20.5 | 20.5 | 20.5 | D | CBA |  |
| ØC (mm) | 12.75±0.15/–0 | 12.75±0.15/–0 | 12.75±0.15/–0 |  |  |  |
| W (mm) | 8.4+1.5/–0 | 8.4+1.5/–0 | 12.4+2/–0 |  |  |  |
| Tmax (mm) | 14.4 | 14.4 | 18.4 |  | MSA284_b |  |
|  |  |  |  |  | W |  |
|  |  |  |  | Fig. 3 Reel dimension | For dimension see Table 6 |  |

PAPER TAPE SPECIFICATION

| Table 7 |  |  |  |  |  | For dimension see Table 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DIMENSION | 0402 | 0603 | 0805 | 1206 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A (mm) | 0.62±0.05 | 1.10±0.05 | 1.65±0.05 | 2.0±0.1 |  |  | T |  |  |  |  |  |  |  |  |  | D 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | P2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B (mm) | 1.12±0.05 | 1.90±0.05 | 2.4±0.05 | 3.5±0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| W (mm) | 8.0±0.2 | 8.0±0.2 | 8.0±0.2 | 8.0±0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | cover tape |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F |  |  |  |  |  |  |  |  |  |  |  |
| E (mm) | 1.75±0.1 | 1.75±0.1 | 1.75±0.1 | 1.75±0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |  |  |  |  |
| F (mm) | 3.5±0.05 | 3.5±0.05 | 3.5±0.05 | 3.5±0.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A |  |  |  |  |  |  |  |  | P1 |  |  |  |  |  |  |  |  |  |  |  |  |  | MBG251_a |  |  |  |  |  |  |
| P0 (mm) | 4±0.05 | 4±0.05 | 4±0.05 | 4±0.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P1 (mm) | 2±0.05 | 4±0.1 | 4±0.1 | 4±0.1 |  | Fig. 4 |  |  |  |  |  | Paper tape dimension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P2 (mm) | 2±0.05 | 2±0.05 | 2±0.05 | 2±0.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ØD0 (mm) | 1.5+0.1 | 1.5+0.1 | 1.5+0.1/–0 | 1.5+0.1/–0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T (mm) | 0.6±0.05 | 0.95±0.05 | 0.95±0.05 | 0.95±0.05 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

BLISTER TAPE SPECIFICATION

| Table 8 |  |  |  |  | For dimension see Table 8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DIMENSION | 0805 | 1206 | 1210 | 1812 |  |  |  |
| A (mm) | 0.20 | 0.30 | 0.30 | 0.40 |  | P0 |  |
|  |  |  |  |  | D 0 | P2 |  |
| B (mm) | 0.20 | 0.30 | 0.30 | 0.40 |  | E |  |
| W (mm) | 8.1±0.2 | 8.1±0.2 | 8.1±0.2 | 12.0±0.2 | pe | F |  |
|  |  |  |  |  |  | W |  |
| E (mm) | 1.75±0.1 | 1.75±0.1 | 1.75±0.1 | 1.75±0.1 |  |  |  |
|  |  |  |  |  |  | B |  |
| F (mm) | 3.5±0.05 | 3.5±0.05 | 3.5±0.05 | 5.5±0.05 |  |  |  |
| P0 (mm) | 4±0.1 | 4±0.1 | 4±0.1 | 4±0.1 |  | MBG516_a |  |
|  |  |  |  |  | A | D 1 |  |
| P1 (mm) | 4±0.1 | 4±0.1 | 4±0.1 | 8±0.1 | T | P1 |  |
| P2 (mm) | 2±0.05 | 2±0.05 | 2±0.05 | 2±0.05 | direction of unreeling |  |  |
| ØD0 (mm) | 1.5+0.1/–0 | 1.5+0.1/–0 | 1.5+0.1/–0 | 1.5+0.1/–0 | Fig. 5 Leader/trailer tape dimension |  |  |
| Tmax (mm) | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

PACKING METHOD

LEADER/TRAILER TAPE SPECIFICATION

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | leader end |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | cover tape only |  |  |
| trailer end |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | empty compartments |  |  |  |  |  | 120 40 mm |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | with cover tape |  |  |  |  |  |  |  |

|  |  | trailer (max. 260 mm) |  |  |  | leader 400 mm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | MEA529 - 2 |  |  |
| Fig. 6 Leader and trailer tape dimension |  |  |  |  |  |  |  |  |  |

METHOD OF MOUNTING

For normal use the capacitors may be mounted on printed-circuit boards or ceramic substrates by applying wave soldering, reflow soldering (including vapor phase soldering) or conductive adhesive in accordance with CECC 00802 classification A.

Typical values (sold line)

Process limits (dotted lines)

| 300 |  | 10 s | MLA859 |  |
| --- | --- | --- | --- | --- |
| T |  |  |  |  |
| (¡C) | 260 ¡C |  |  |  |
| 250 | 245 ¡C |  |  |  |
|  | 215 ¡C | 10 s |  |  |
| 200 |  | 40 s |  |  |
|  | 180 ¡C |  |  |  |

150

130 ¡C

100

2 K/s

50

0

0	50 100 150 200 250 t (s)

Fig. 7	Recommended reflow soldering profile

Typical values (sold line)

Process limits (dotted lines)

The capacitors may be soldered twice in accordance with this method if desired

300

10 s

| T |  |  |  |  | MLA861 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (¡C) |  |  |  |  |  |  |  |
| 250 | 235 ¡C to 260 ¡C |  | second wave |  |  |  |  |
| 200 | first wave |  | 5 K/s |  |  |  |  |
|  |  |  | 2 K/s |  |  |  |  |
| 150 | 200 K/s |  |  |  |  |  |  |
|  | 100 ¡C to 130 ¡C | forced |  |  |  |  |  |
| 100 |  |  |  |  |  |  |  |
|  |  | cooling |  |  |  |  |  |
| 50 |  | 2 K/s |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |
| 0 | 50 | 100 | 150 | 200 | t (s) | 250 |  |

Fig. 8	Recommended wave soldering profile

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  |  | 9 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V | 10 |  |  |  |

TEST AND REQUIREMENT

| Table 9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| IEC384-10 | TEST ITEMS | CONDITIONS | REQUIREMENTS |  |
| 4.9 | Bending | Bending rate 1mm/s, jig. radius 340mm | ∆C/C≤10% |  |
| 4.10 | Resistance to soldering heat | 260±5°C for 10±0.5s in static solder | –5%≤∆C/C≤10% |  |
|  |  | bath |  |  |
| 4.11 | Solderability | 235±5°C for 2±0.5s in a static solder | 75% minimum coverage of metallic area |  |
|  |  | bath |  |  |
| 4.12 | Rapid change of temperature | Preconditioning | ∆C/C within 15% |  |
|  |  | –55°C to +125°C, 5cycles |  |  |
|  |  | Preconditioning | ∆C/C within 15% |  |
| 4.14 | Damp heat | At 40°C, 90 to 95% RH and Ur applied | Tan δ≤7% |  |
|  |  | (max. 500V) for 500 hours | IR≥500MΩ or RxC≥25s whichever is less |  |

|  |  | Preconditioning |
| --- | --- | --- |
| 4.15 | Endurance | 2xUr applied for 1,000 hours, at upper |
|  |  | category temperature |

∆C/C within 20%

Tan δ≤7%

IR≥1,000MΩ or RxC≥50s whichever is less

www.yageo.com Aug 17, 2005 V.9

|  |  |  |  |  | 10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Surface Mount Multilayer Ceramic Capacitors | CC | SERIES | X7R, 16 V to 100 V |  | 10 |  |  |

REVISION HISTORY

| REVISION | DATE | CHANGE NOTIFICATION | DESCRIPTION |
| --- | --- | --- | --- |
| Version 9 | Aug 17, 2005 | - | - 0603 50V capacitance range extended to 100 nF |
| Version 8 | Jan 21, 2005 | - | - Capacitance range extended |
| Version 7 | Feb. 24, 2004 | - | - upgrade the thickness on page 3-4 |
| Version 6 | Aug. 13, 2003 | - | - Taping drawing amended |
| Version 5 | Jun. 19, 2003 | - | - Thickness of 1206/470 nF revised into “1.15±0.1” |
|  |  |  | - CTC code size 1812 revised |
| Version 4 | Jan. 27, 2003 | - | - Edit “SCOPE” |
| Version 3 | Dec. 18, 2002 | - | - New Yageo edition |
| Version 2 | Jul. 04, 2002 | - | - |

www.yageo.com Aug 17, 2005 V.9